Root Flow Plant Health Sensor

Hardware Build Guide

Hamza M Ali

# Section 1 — Project Concept

Most plant sensors are single-device: one sensor in the soil, one app. Root Flow treats the plant as two separate microenvironments — root zone and canopy — and monitors both independently.

The Soil Pod lives at the base of the plant. It measures:

- Soil moisture, temperature, electrical conductivity (EC), and pH via the ALOUTSNOC RS485 Modbus probe

- Full visible-spectrum light (415–680 nm + NIR + Clear) via an AS7341 11-channel spectral sensor

The Leaf Pod sits higher on the plant or nearby. It:

- Receives processed sensor data from the Soil Pod over the inter-pod RS485 bus

- Connects to your home Wi-Fi

- Sends Telegram messages when thresholds are breached

- Runs a 12V boost converter to supply RS485 bus power

RS485 was chosen for the inter-pod link because it tolerates cable runs up to 1200 m, is immune to ground-loop noise, and requires only two wires (A and B). The Nano operates at 5V logic; the ESP32-CAM operates at 3.3V. A DEVMO IIC bidirectional logic level converter between them prevents the ESP32-CAM's GPIO pins from being damaged by 5V signals.

The Soil Pod uses a second MAX485 module and SoftwareSerial to communicate with the ALOUTSNOC Modbus probe separately from the inter-pod bus. This keeps the two RS485 channels independent and avoids address conflicts.

# Section 2 — Parts List

## Soil Pod Components

| Qty | Part | Purpose | Approx. Cost | Where to Buy |
| --- | --- | --- | --- | --- |
| 1 | Arduino Nano (ATmega328P) | Soil pod microcontroller | $4–8 | Amazon, AliExpress, Arduino.cc |
| 1 | ALOUTSNOC RS485 Modbus Soil Sensor | Soil moisture, temp, EC, pH via RS485 Modbus RTU | $15–25 | Amazon (ALOUTSNOC listing) |
| 1 | AS7341 11-Channel Visible Spectrum Sensor | Full-spectrum light (415–680 nm + NIR + Clear) | $8–12 | Adafruit #4698, Amazon |
| 2 | SJZBIN MAX485 Transceiver Module | One for inter-pod RS485 bus; one for Modbus soil probe | $1–2 ea. | Amazon (SJZBIN 5PCS pack) |
| 1 | 4.7 kΩ resistor | Spare / pull-up if needed | <$0.10 | Local electronics / kit |
| 2 | 100 nF ceramic capacitor | MAX485 decoupling (one per module) | <$0.10 ea. | Local electronics / kit |
| 1 | Breadboard or proto board (half-size) | Mounting | $2–4 | Amazon |
| — | Jumper wires (M-M and M-F) | Connections | $3–5 (set) | Amazon |

## Leaf Pod Components

| Qty | Part | Purpose | Approx. Cost | Where to Buy |
| --- | --- | --- | --- | --- |
| 1 | ESP32-CAM (AI-Thinker, Built-in USB-C TTL) | Leaf pod microcontroller + Wi-Fi. The built-in USB-C TTL version eliminates the need for a separate FTDI programmer. | $8–14 | Amazon (ESP32 Cam kit with built-in USB-C TTL) |
| 1 | SJZBIN MAX485 Transceiver Module | Inter-pod RS485 bus interface | $1–2 | Amazon (SJZBIN 5PCS pack) |
| 1 | DEVMO IIC I2C 4-Channel Logic Level Converter (3.3V to 5V) | Protect ESP32-CAM GPIO from 5V MAX485 signals | $1–3 | Amazon (DEVMO listing) |
| 1 | MT3608 DC-DC Boost Module (12V output) | Power RS485 bus termination | $1–3 | Amazon, AliExpress |
| 2 | 120 Ω resistors | RS485 bus termination (one per pod end) | <$0.10 ea. | Local electronics / kit |
| 1 | Breadboard or proto board (half-size) | Mounting | $2–4 | Amazon |
| — | 2-wire twisted pair cable (≥ 1 m) | RS485 A/B inter-pod bus | $3–5/m | Amazon, local hardware |

## Shared / Power

| Qty | Part | Purpose | Approx. Cost |
| --- | --- | --- | --- |
| 1 | diymore 18650 Battery Holder | Holds 18650 cell for Leaf Pod | $1–3 |
| 1 | 18650 LiPo Cell (3.7V) | Leaf Pod battery | $4–8 |
| 1 | diymore TP4056 USB-C Charging Module (from 10PCS pack) | Charges 18650 cell; provides 5V output to Leaf Pod | $1–2 |
| 1 | Micro USB 5V power supply (≥ 500 mA) | Soil Pod power | $5–8 |

## Bench / Test Tools (not wired into circuit)

| Part | Purpose |
| --- | --- |
| diymore USB Tester | Measure USB supply voltage and current draw during bench testing |
| diymore Type-C Meter (Voltage / Current) | Monitor TP4056 charge current and Leaf Pod power consumption in real time |

NOTE:  The USB tester and Type-C meter are diagnostic tools only. They sit between power supply and device during testing and are removed for final deployment.

# Section 3 — Wiring

Work through wiring in five phases. Complete and verify each phase before moving to the next. Use a multimeter to confirm continuity on every ground connection before applying power.

## Phase 1 — Soil Pod: AS7341 Spectral Sensor → Arduino Nano

The AS7341 replaces the TCS34725 from earlier revisions. It communicates over I2C and shares the bus with any other I2C devices on the Nano.

AS7341 VIN  →  Nano 3.3V   (breakout regulates internally to 1.8V)

AS7341 GND  →  Nano GND

AS7341 SDA  →  Nano A4    (I2C Data)

AS7341 SCL  →  Nano A5    (I2C Clock)

AS7341 LDR  →  Not connected (leave floating unless using onboard LED)

NOTE:  The AS7341 I2C address is fixed at 0x39. Install the Adafruit AS7341 library via Arduino IDE Library Manager. The breakout's onboard regulator accepts 3.3V–5V on VIN.

## Phase 2 — Soil Pod: ALOUTSNOC RS485 Modbus Soil Probe → Arduino Nano

The ALOUTSNOC probe speaks RS485 Modbus RTU. It needs its own MAX485 module on the Nano connected via SoftwareSerial — this keeps it completely separate from the inter-pod RS485 bus that uses the Nano's hardware UART (D0/D1).

The probe cable colors can vary by batch. Check your included datasheet. Common mapping:

Probe Brown (VCC)  →  12V rail  (from MT3608 boost output on Leaf Pod,

or a separate 12V supply if testing standalone)

Probe Black (GND)  →  Common GND

Probe Yellow (A+)  →  Soil-side MAX485 module  A pin

Probe Blue  (B-)   →  Soil-side MAX485 module  B pin

Soil-side MAX485 (Modbus probe bus) → Arduino Nano via SoftwareSerial:

MAX485-SOIL VCC  →  Nano 5V

MAX485-SOIL GND  →  Nano GND

MAX485-SOIL DI   →  Nano D6   (SoftwareSerial TX)

MAX485-SOIL RO   →  Nano D5   (SoftwareSerial RX)

MAX485-SOIL DE   →  Nano D3   (Direction control — tie DE and RE together)

MAX485-SOIL RE   →  Nano D3   (tied to DE)

MAX485-SOIL A    →  Probe A+ wire

MAX485-SOIL B    →  Probe B- wire

CRITICAL — The probe requires 5–30V on its VCC wire. Do NOT connect it to the Nano 5V pin — it will brown-out the board. Power it from the MT3608 12V output (shared with the bus) or a dedicated 12V wall supply. Share only GND with the Arduino circuit.

NOTE:  Place a 100 nF ceramic capacitor across VCC and GND of this MAX485 module to suppress switching noise from the Modbus polling.

## Phase 3 — Soil Pod: Inter-Pod RS485 Bus MAX485 → Arduino Nano

This is the second MAX485 on the Nano — used exclusively for sending data to the Leaf Pod. It uses the Nano's hardware UART (D0/D1).

MAX485-BUS VCC  →  Nano 5V

MAX485-BUS GND  →  Nano GND

MAX485-BUS DI   →  Nano D1  (Hardware TX)

MAX485-BUS RO   →  Nano D0  (Hardware RX)

MAX485-BUS DE   →  Nano D2  (Direction control — tie DE and RE together)

MAX485-BUS RE   →  Nano D2  (tied to DE)

MAX485-BUS A    →  Bus Wire A (twisted pair to Leaf Pod)

MAX485-BUS B    →  Bus Wire B (twisted pair to Leaf Pod)

CRITICAL — The Nano's D0/D1 are shared with USB programming. Disconnect the inter-pod MAX485 wires from D0/D1 before uploading sketches. Reconnect after upload.

NOTE:  Place a 100 nF ceramic capacitor across VCC and GND of this MAX485 module as well.

## Phase 4 — Leaf Pod: ESP32-CAM Wiring

The ESP32-CAM (AI-Thinker) has most of its GPIO pins consumed by the camera and PSRAM. The safe free pins available for this project are: GPIO 2, GPIO 13, GPIO 14, GPIO 15. I2C is not natively broken out; it must be assigned via TwoWire in code.

Inter-Pod MAX485 Module → DEVMO Logic Level Converter → ESP32-CAM:

Level Shifter Power Rails:

DEVMO HV pin  →  5V rail  (from TP4056 OUT+ / Vin pin on ESP32-CAM)

DEVMO LV pin  →  ESP32-CAM 3.3V pin

DEVMO GND     →  GND (both sides share common GND)

Signal Lines Through DEVMO Shifter Channels:

ESP32-CAM GPIO 13  →  DEVMO LV Ch1  →  DEVMO HV Ch1  →  MAX485 DI

ESP32-CAM GPIO 2   →  DEVMO LV Ch2  →  DEVMO HV Ch2  →  MAX485 RO

ESP32-CAM GPIO 15  →  DEVMO LV Ch3  →  DEVMO HV Ch3  →  MAX485 DE + RE (tied)

MAX485 Bus Connections:

MAX485 VCC  →  5V rail (ESP32-CAM Vin)

MAX485 GND  →  GND

MAX485 A    →  Bus Wire A (twisted pair from Soil Pod)

MAX485 B    →  Bus Wire B (twisted pair from Soil Pod)

CRITICAL — Never connect the ESP32-CAM GPIO pins directly to 5V MAX485 signals. The ESP32-CAM is strictly 3.3V logic. Always route through the DEVMO level shifter.

CRITICAL — GPIO 12 on the ESP32-CAM is a bootstrapping pin that controls flash voltage. Do NOT use it for I2C or any signal — connecting it LOW at boot can corrupt the internal voltage regulator. It is reserved and must remain floating.

Programming Note (Built-in USB-C TTL version):

Your ESP32-CAM kit includes built-in USB-C TTL, so no external FTDI programmer is needed. To flash: hold the BOOT button while plugging in USB-C, release after 1 second, then upload from Arduino IDE. After flashing, press the EN (reset) button to run normally.

## Phase 5 — Power Wiring (Leaf Pod — Battery)

The Leaf Pod runs on an 18650 LiPo cell in the diymore battery holder, charged by the TP4056 USB-C module.

Battery to TP4056:

TP4056 B+   →  18650 positive terminal (via diymore holder red wire)

TP4056 B-   →  18650 negative terminal (via diymore holder black wire)

TP4056 to ESP32-CAM:

TP4056 OUT+  →  ESP32-CAM 5V / Vin pin

TP4056 OUT-  →  ESP32-CAM GND

5V Rail Distribution (from ESP32-CAM Vin):

ESP32-CAM Vin  →  MAX485 VCC

ESP32-CAM Vin  →  DEVMO HV pin

ESP32-CAM Vin  →  MT3608 IN+

Charging:

TP4056 USB-C port  →  5V USB-C charger (can remain connected during operation)

NOTE:  Use the diymore Type-C meter inline between the USB-C charger and TP4056 during initial setup to monitor charge current. Remove it for permanent installation.

The Soil Pod is powered separately by its own Micro USB 5V supply, or by the 12V probe supply sharing GND. This keeps ground planes clean.

## Phase 6 — RS485 Inter-Pod Bus and MT3608 Boost Converter

Bus Connections

Soil Pod MAX485-BUS A  →  [twisted pair]  →  Leaf Pod MAX485 A

Soil Pod MAX485-BUS B  →  [twisted pair]  →  Leaf Pod MAX485 B

Use a twisted pair from a Cat5/Cat6 ethernet cable if dedicated RS485 cable is unavailable. Label each end to avoid swapping A and B.

Termination Resistors

Soil Pod side:  120 Ω between Bus A and Bus B

Leaf Pod side:  120 Ω between Bus A and Bus B

NOTE:  At 9600 baud on cables under 5 m you may not notice errors without terminators. Install them now so the system is upgradeable to higher baud rates.

MT3608 Boost Converter Tuning

CRITICAL — The MT3608 ships with an arbitrary preset voltage. You must tune it before connecting it to anything.

- Connect MT3608 IN+ to 5V rail and IN- to GND. Leave output disconnected.

- Place multimeter probes on MT3608 OUT+ and OUT-.

- Slowly turn the potentiometer clockwise while reading the voltage.

- Stop at 12.0 V +/- 0.2 V.

- Connect OUT+ to the RS485 bus bias network and to the ALOUTSNOC probe VCC. Connect OUT- to GND.

MT3608 IN+   →  5V rail (Leaf Pod Vin)

MT3608 IN-   →  GND

MT3608 OUT+  →  12V output to Soil Pod probe VCC + RS485 bias

MT3608 OUT-  →  GND

# Section 4 — Critical Hardware Callouts

CRITICAL — ESP32-CAM GPIO Voltage: The ESP32-CAM is a 3.3V device. Its GPIO pins are not 5V tolerant. Any direct connection from a 5V MAX485 output will permanently damage the chip. Always route through the DEVMO IIC logic level converter.

CRITICAL — GPIO 12 Reserved on ESP32-CAM: This pin controls the internal flash voltage regulator bootstrapping. Never connect it to any signal. Leave it floating.

CRITICAL — ALOUTSNOC Probe Power: The probe requires 5–30V (typically 12V for reliable readings). Never power it from the Nano 5V pin — the inrush current will brown-out the board and may damage the sensor electronics. Always use the MT3608 12V output or a dedicated 12V supply.

CRITICAL — Two Separate RS485 Channels on the Nano: The Nano runs two independent MAX485 modules. MAX485-SOIL handles the Modbus probe on SoftwareSerial (D5/D6/D3). MAX485-BUS handles the inter-pod link on HardwareSerial (D0/D1/D2). These must never be bridged.

CRITICAL — Disconnect D0/D1 Before Uploading to Nano: The hardware serial pins (D0/D1) are shared with the USB programmer. Disconnect MAX485-BUS from D0 and D1 before uploading any sketch. Reconnect after upload is complete.

CRITICAL — Boost Converter Pre-Tuning: Set the MT3608 output to 12V before connecting it to any circuit. Untuned, it may output 24V or more, destroying the probe and any connected device instantly.

CRITICAL — RS485 Termination: Install 120 Ohm resistors at both ends of the inter-pod bus now. At 9600 baud over short runs you may not notice errors, but missing terminators will cause data corruption at higher baud rates.

# Section 5 — Hardware Validation Checklist

Run through all checks with a multimeter before connecting to a computer or uploading code.

- Soil Pod ground continuity: Confirm GND is shared between Nano, AS7341, MAX485-SOIL, and MAX485-BUS. In continuity mode, probe each GND pin against Nano GND — should beep on all.

- AS7341 I2C address confirmed: After uploading an I2C scanner sketch, the monitor should show device at 0x39. If nothing appears, re-check A4/A5 wiring and confirm VIN is on 3.3V.

- ALOUTSNOC probe power: With MT3608 tuned and connected, measure voltage on the probe VCC wire. Should read 12.0 V +/- 0.5 V. Probe will not respond below 5V.

- SoftwareSerial Modbus response: Upload a Modbus RTU poll sketch. The Serial Monitor should return soil readings within 2–3 seconds. No response usually means A/B wires are swapped — try reversing them.

- DEVMO level shifter orientation: Confirm LV side faces ESP32-CAM (3.3V) and HV side faces MAX485 (5V). Reversed installation produces no communication but causes no damage.

- Boost converter output measured: With only input power connected (no load), confirm output reads 12.0 V +/- 0.2 V on a multimeter before connecting to the bus or probe.

- RS485 termination resistors installed: With power off, measure resistance across Bus A and Bus B at each pod end. Should read ~60 Ohm (two 120 Ohm in parallel).

- ESP32-CAM flashes and boots: Using the built-in USB-C TTL, upload a blink sketch. Hold BOOT during plug-in, then press EN after upload. LED should blink. Confirms USB-C programmer is working.

# Section 6 — Hardware Troubleshooting Matrix

| Symptom | Most Likely Cause | Diagnostic Step | Fix |
| --- | --- | --- | --- |
| AS7341 not found on I2C scan (no 0x39) | Wrong pins or VIN too low | Confirm A4=SDA, A5=SCL on Nano; measure VIN pin (~3.3V) | Re-seat I2C wires; ensure VIN on 3.3V not 5V |
| ALOUTSNOC probe returns no data | A/B wires swapped or probe not powered | Measure probe VCC wire — should be 12V. Try swapping A and B wires. | Reverse A/B wires; verify MT3608 output is 12V |
| Modbus poll times out (no response) | Wrong baud rate or SoftwareSerial pin mismatch | Confirm D5=RO, D6=DI, D3=DE/RE in code; set baud to 9600 | Re-check SoftwareSerial pin assignments; try 4800 baud as fallback |
| No RS485 inter-pod communication | DEVMO level shifter reversed or GPIO mismatch | Probe Bus A — should swing 0V to 5V during transmit | Check LV/HV orientation on DEVMO; confirm GPIO assignments in ESP32-CAM code |
| ESP32-CAM won't enter flash mode | GPIO 0 not pulled low during boot | Hold BOOT button while connecting USB-C; release after 1 second | Built-in USB-C TTL version handles this automatically — try EN (reset) button after upload |
| Boost converter output too high/low | Potentiometer not tuned | Measure MT3608 output with multimeter, nothing else connected | Adjust pot slowly; stop at 12.0 V +/- 0.2 V |
| Soil Pod Nano browns out on power-up | Probe drawing current through Nano 5V | Disconnect probe VCC; if Nano boots normally, probe is incorrectly powered | Move probe VCC to MT3608 12V output; share only GND with Nano |
| Two readings conflict (I2C hang) | AS7341 and MAX485-SOIL sharing SoftwareSerial on wrong pins | Confirm AS7341 is on A4/A5 (I2C) and MAX485-SOIL is on D5/D6 (SoftwareSerial) | Re-assign pins so I2C and SoftwareSerial do not overlap |